PROJECT: Emergency Network LAN

====================================

1) IP ADDRESSES

====================================

Router R1

- FastEthernet0/0 = 192.168.1.1/24

- FastEthernet1/1 = 192.168.2.1/24

AlpineLinux-2

- IP = 192.168.1.10/24

- Gateway = 192.168.1.1

AlpineLinux-3

- IP = 192.168.1.20/24

- Gateway = 192.168.1.1

AlpineLinux-1 (Server)

- IP = 192.168.2.10/24

- Gateway = 192.168.2.1

Networks

- Users Network: 192.168.1.0/24

- Server Network: 192.168.2.0/24

====================================

2) ROUTER CONFIGURATION

====================================

enable

configure terminal

interface f0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

interface f1/1

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

end

copy running-config startup-config

====================================

3) ACL CONFIGURATION

====================================

Goal:

- Allow AlpineLinux-2 to access the server

- Block AlpineLinux-3 from accessing the server

enable

configure terminal

access-list 100 deny ip host 192.168.1.20 host 192.168.2.10

access-list 100 permit ip any any

interface f0/0

ip access-group 100 in

end

copy running-config startup-config

====================================

4) ALPINELINUX-2 CONFIGURATION

====================================

/start.sh

Check IP:

ip a

Expected:

- eth0 = 192.168.1.10/24

Test router:

ping 192.168.1.1

Test server access:

nc 192.168.2.10 80

Chat listen mode:

nc -l -p 1234

====================================

5) ALPINELINUX-3 CONFIGURATION

====================================

/start.sh

Check IP:

ip a

Expected:

- eth0 = 192.168.1.20/24

Test router:

ping 192.168.1.1

Test server access (should be blocked by ACL):

nc 192.168.2.10 80

Chat connect mode:

nc 192.168.1.10 1234

====================================

6) SERVER CONFIGURATION

====================================

/start.sh

Check IP:

ip a

Expected:

- eth0 = 192.168.2.10/24

Test router:

ping 192.168.2.1

Simple Emergency Page using netcat:

nc -l -p 80

Displayed content:

<h1>EMERGENCY NETWORK</h1>

<p>Stay Calm</p>

<p>Evacuate immediately</p>

<p>Call 911</p>

====================================

7) TESTING

====================================

A) Routing Test

From AlpineLinux-2:

ping 192.168.2.10

Result:

- Success

B) Chat Test

On AlpineLinux-2:

nc -l -p 1234

On AlpineLinux-3:

nc 192.168.1.10 1234

Result:

- Chat between two users works successfully

C) ACL Test

From AlpineLinux-2:

nc 192.168.2.10 80

Result:

- Access allowed

From AlpineLinux-3:

nc 192.168.2.10 80

Result:

- Access blocked

====================================

8) PROJECT FEATURES

====================================

- Local Area Network (LAN)

- Routing between two subnets

- Emergency server

- Chat system between clients

- Access Control List (ACL) security